LABORATORY EXPERIMENTAL PROTOCOL

Applied Massive Parallelism: CUDA Python, Thread Geometries, and Hardware Latency Pitfalls

Student Name: Bekbolatuly Yernar

Student ID: 230109011

Session / Date: 08.10.2026

Assigned GPU ID: GPU 0 — Tesla T4 (Bus-Id 00000000:00:04.0)

# SECTION 1: Hardware Telemetry (Task 1.1)

| Field | Recorded value |
| --- | --- |
| NVIDIA Driver Version (from nvidia-smi table) | 580.82.07 |
| CUDA Version Supported by Driver | 13.0 |
| Allocated GPU Name | Tesla T4 |
| Total Available VRAM | 15360 MiB (nvidia-smi); 14913 MiB reported by Numba |
| Compute Capability Major.Minor | 7.5 |
| Max Threads Per Block (hardware limit) | 1024 |

# SECTION 2: The Crossover Experiment

Q1. Empirical Crossover Point N*:

N* = 10,000. At N = 1,000 the CPU was still faster (0.2412 ms vs 0.5880 ms GPU Total). At N = 10,000 the GPU Total time (0.4048 ms) became lower than the CPU time (2.4196 ms), and the gap kept growing for larger N.

Q2. Why is GPU Total slower than CPU at N = 100?

At N = 100 the GPU Total time was 0.6259 ms, while the GPU Kernel Only time was just 0.0630 ms. About 89.9% of the elapsed time ((0.6259 − 0.0630) / 0.6259) was overhead rather than computation: allocating device memory, launching the kernel, and copying data host-to-device and device-to-host over the high-latency PCIe bus. The data is so small that this fixed cost dwarfs the actual computation, so the CPU (0.0799 ms) wins.

# SECTION 3: Thread Geometry & Boundary Corruption

## Task 3.1: Observed behavior / Console output from defective kernel

Launching defective kernel with 1024 threads on 1000-element array...

Execution finished (or did it crash?). Checking host copy...

Elements processed: 1000. Last element: 999.0

Only a NumbaPerformanceWarning (grid size 4, low occupancy) was shown. No Python IndexError and no CUDA_ERROR_ILLEGAL_ADDRESS appeared: threads 1000–1023 wrote outside the 1000-element array silently, because GPU memory is raw VRAM with no bounds checking. The corruption of adjacent memory went undetected, which is the danger of a missing 'if idx < n' guard.

## Task 3.2: Mandatory calculation (N = 37, Block Size = 8)

| Quantity | Value |
| --- | --- |
| Total Blocks Allocated | 5 |
| Total Hardware Threads Launched | 40 |
| Number of Active Working Threads | 37 |
| Number of Idle Guarded Threads | 3 |

Block 4 boundary: global IDs 32–36 were ACTIVE; global IDs 37, 38, 39 (thread IDs 5, 6, 7) were IDLE (GUARDED).

# SECTION 4: Race Condition (Parallel Reduction)

Atomic Sum Output: 50000.0 — equals exactly 50,000.0? YES

Naive Kernel Time: 0.0941 ms

Atomic Kernel Time: 0.2633 ms

Why do atomic operations incur a performance penalty?

Atomic operations force the hardware to serialize updates to the same memory address: each thread must wait for the previous read-modify-write on d_total[0] to complete, so thousands of threads queue up instead of running in parallel. The naive kernel is faster (0.0941 ms vs 0.2633 ms, about 2.8x) but wrong, because simultaneous read-modify-write sequences overwrite each other and almost all updates are lost (result 2.0 instead of 50,000.0).

# SECTION 5: 2D Grid Processing (1024 × 1024)

| Quantity | Value |
| --- | --- |
| Blocks in X | 64 |
| Blocks in Y | 64 |
| Total Blocks | 4096 |
| Verified pixel value at center (512, 512) | 1.0000 |
| Verified pixel value at outer corner (0, 0) | 0.0000 |

# SECTION 6: Final Verification Token

TOKEN: F71E22F5C3F8FBF8